Copyright 1992-1993 by Patrick Cook Resources,Inc.

Welcome  to NBS Trig, the most intuitive  SHOP MATH program available.

NBS Trig was designed to solve SHOP MATH problems quickly, without

forcing the user to spend days studing a user manual to become competent

with its abilities.

If  you   have  a  Hercules video card  you must  copy  the  file

MSHERC.COM  to  your  root  directory  and  install  it  in   the

AUTOEXEC.BAT file and  reboot the system.

NBS Trig runs on XT/AT compatible computers with VGA,EGA, CGA  or

MDA video capabilities and 450 Kbytes of conventional memory.

To start NBS Trig enter:

C:nbs203<cr>

The  MAIN menu is composed of eight pictures that depict  the  SHOP

MATH FUNCTIONS that can be solved, as follows:

TRIANGLES - Will solve RIGHT, ACUTE or OBTUSE triangles

For RIGHT triangles enter one side and  one

angle  or two sides.  For ACUTE  or  OBTUSE

triangles enter one angle and two sides  or

two angles and one side or three sides.

BALL  CHECKS - Will solve a variety of problems  with  two

balls in a tapered hole. The LARGEST and/or

SMALLEST  balls that will fit in a  tapered

hole. The length from the top of two  balls

in  a tapered hole.  Given two balls  in  a

tapered hole, NBS Trig will solve for SHARP

CORNERS   or   INCLUDED   ANGLE   or   BALL

SUBSTITUTION.  It will also calculate  BALL

DROPS.

BOLT HOLES - Will calculate EQUALLY SPACED HOLES  around

a circle of from 3 to 999 holes.  Holes can

be REFERENCED DEAD CENTER ,TANGENTIALLY  or

by DATUM  point.   The first  hole  can  be

located by ANGLE entry, TOP DEAD CENTER  or

LEFT  OF  TOP  DEAD  CENTER.    INCREMENTAL

location of equal spaced holes can also  be

calculated.

SINE-BARS  - Will    calculate  ANGLE,   HEIGHT   and/or

COMPOUND     TAPER per FOOT, given the  entry of one  of

SINE-PLATES  the  above, for single and double  sine-bar

problems. Also, ANGLES and DIMENSION of

compound sine-plates and rotated compound

sine-plates.

CHORDS  - Given  any  two of the  following:  ANGLE,

HEIGHT,  RADIUS,  CHORD  or   ARC  LENGTH,

will  solve  for  the  others  and  CIRCLE

TANGENTS.

COMPOUND ANGLES  - Selecting   COMPOUND  ANGLE  will   provide

solutions  for  five  of  the  most  common

problems.

DOVETAILS - Will   solve   ANGLE  or SHARP  CORNERS  or

CONE CHECKS   DIMENSION-OVER-ROLLS ( DOR )  for  MALE and

FEMALE dovetails or CONES.

SPEED, FEED & - Provides two ways of finding speed and feed

HORSEPOWER      values. The QUICK solution requires the user

to supply all data neccescary for the cut.

The DETAILED solution allows the user to make

a series of menu selection that describe the

operation (i.e., milling, turning or drilling)

tool type, material and cutter material.

Horsepower requirements are also calculated for

the cut.  Maximum speed and feed ( roughing )

values are calculated based on the machines

horsepower  and type of drive.

FORMULA CALCULATOR - The built in formula solving calculator can be

used three seperate ways:

1)  At any data enter line a formula can be

entered in leau of an angle or dimension.

During the solution of the problem the

formula will be solved and the result used

as the data required for the angle or

dimension.

2)  Except in the FORMULA module, pressing F5

will popup the calcultor in the lower part

of the display.

3)  Selecting the FORMULA module from the MAIN

MENU allows the user to create or delete

formula libraries.  Within the libraries

formulas can be entered, modified, deleted

and/or saved to disk.

The registration fee is $65.00.  After registration a manual, registration

number and a phone number for technical assistance will be issued.  Make

checks payable to:

Patrick Cook Resources, Inc.

1550 Dell Ave., Suite C

Campbell, CA  95008