AVIXIVA Reflective Systems

20 Nov 93

DA-3 AudioSoft

Chromium Lizard Version

Operating Guide

DA-3 is audio play software designed to work with Hitachi bus CD-ROM

drives on PC/AT bus computers running MS-DOS and using a color display.

DA-3 is Hardware Direct and requires no device drivers or MS-DOS CD-ROM

Extensions software.  The program talks directly to the CD-ROM hardware

and can thus perform operations not supported by the Extensions or Hitachi

device drivers.  This means that you must have one of the interface cards

and drives required by the program.  Hardware functions other than CD-ROM

are handled by the programming language used to develop the program.

Features

*   The Microsoft CD-ROM extensions and/or drivers need not be loaded to

play audio tracks (extensions-free software)

*   Support for all Hitachi bus drives equipped with headphone jack.

*   Any of up to 8 drives per interface card can be used for audio play.

*   Any one of eight interface card I/O port addresses can be used and

the address can be easily set from DOS without re-booting.

*   Automatic card address search when address not set, address verification

and invalid address display with help message

*   Automatic active drive seeking at startup or after selection of an

inactive drive ID.

*   Automatic play continuation at startup after reading maximum tracks

and times (after first minute of play).

*   Automatic repeat play of entire disc

*   Play by starting and ending track number

*   Smart and easy track number input

*   Forward/Reverse by Track, minute or 10 second steps with beginning

and end of disc wraparound.

*   Restart current track

*   Cue to current track start with pause

*   Direct SlideCue access to any starting minute

*   Play time scale and play location indication

*   Display of maximum track and time, or remaining time

*   Selectable display of disc elapsed time or track elapsed time

*   Repeat play by track or section with optional display modes

*   Play from marked position

*   Que play from a preset, random or shuffled track list.

*   Pause mode with on screen indication

*   Stereo or left/right channel selection and indication

*   Active drive indication, selected drive indication, drive model

indication with optional display, and 4/8 drive interface type indication.

*   Eject (all models) and Accept (CDR1503S) with indication for disc

installed.

*   Screen color selection from 64 paper and ink colors

*   Presetable startup colors, in-program paper color selection and color

preset restore

*   Automatic drive locking with unlock option.

*   Automatic Sleep mode disable/enabled with Stop

*   Laser level operation check - CDR1600S 1700S 1900S 3600 3700 6700

*   Drive Firmware display

*   Startup default drive selection

*   Operates in DOS window of OS/2

*   Help screen

*   The program is non-resident when exited.  It occupies memory only when

active and does not pop-up nor operate in the background unless it is set

for background operation under Windows. When play is started, and the

program is exited, any of the remaining CD-ROM drives can be used for

CD-ROM applications.

Supported CD-ROM Drives and Interface Cards:

Hitachi Drives:

CDR101AS CDR1503S CDR1600S CDR1700S/SA CDR1900S

CDR101A  CDR3500  CDR3600  CDR3700     CDR6700

Hitachi bus compatible drives include Todd, Amdek and

Sun Moon Star models.

(SCSI Bus drives not supported:  CDR1750S CDR3650 CDR3750)

Interface Cards:

CDIF25A2, CDIF25A3, CDIF35A, CDIFI4/A, CDIFI8A/S

and compatible PC Bus to Hitachi bus interface cards

The Micro Channel bus and PC bus to SCSI bus interface cards

are not supported (CDIFI5A, TMC845, TMC850M MCS600).

Computers:

IBM hardware compatible computers which use 8 bit or 16 bit

PC bus expansion slots:  XT-AT, PS/2 25-40, AT class 486DX2/50MHz..

Installation:

Copy the program to your hard disk or to a floppy.  On a hard disk,

put DA-3 into one of the subdirectories listed in your PATH statement

or into whatever directory you want to be in when you use it.

Turn on the CD-ROM drive, connect a set of mini-stereo plug headphones to

the headphone jack, turn up the volume halfway, and insert an audio compact

disc into the disc caddy (or tray) and drive.

Start the program by typing the program name and pressing enter:  DA3

DA3 will load and automatically scan the addresses for the Hitachi interface

card in sequence of 300 360 340 320 260 240 220 200H.  Any CDR3700 or

TCDR7000 drive must be turned on or the card will not be identified by

reading the port addresses.  If you have the correct card and DA3 does not

find it, you must establish the address from DOS with SET AVIX=xxx where

xxx is the address.  The address scan feature is not foolproof.  Some

computers and interface cards give different results when reading empty

or occupied addresses.  Scan is disabled when one of the valid addresses

is supplied to with the AVIX= variable.

When the program does not see the interface card at the address it

will halt and indicate that you need to establish the address in DOS using

the SET command of DOS.  Make sure that all drives are turned on when

trying to verify the address.

SET AVIX=340

SET AVIX=360

etc...according to the address jumper setting on your interface card.

CHECKING THE CARD ADDRESS

If DA3 locates the card and starts working, press H for the help screen.

The address used is shown at the top left when AVIX=xxx is not specified.

If you have our port check utility PCHECK.EXE run it to determine what

address is in use, or if you have the MS-DOS Extensions or the driver for

the extensions installed, type the CONFIG.SYS file to the screen.  Look for

the line which has the HITACHIA.SYS driver.  At the end of that line is

/P:xxx  where xxx is a 3 digit address such as 300, (or 200, 220, 240, 260,

300, 320, 340, 360, which matches address jumper settings S0-S7).  This is

the address used by the interface card and driver when the CD-ROM drive is

operating.  If the line does not have a /P:300 or similar, the card is

using the default setting of 300.

Example CONFIG.SYS line for Hitachi extensions driver:

DEVICE=C:\DEV\HITACHIA.SYS /D:MSCD000 /N:1 /P:300

If the line has HITACHIB.SYS and the CD-ROM drive has been working,

do not use this program.  You need to use a program which works through

the MS-DOS extensions and drivers. If you do not have any driver line with

HITACHIA.SYS or HITACHI.SYS, your existing software may be hardware direct

at 300H or your driver has another name for the file and /D:xxxxxx.

If you don't know what address the card is set for, you can try each setting

in sequence.  Use the SET command and verify that you typed the name and

address correctly (enter SET without any parameters), and restart the

program.  In the unlikely event the computer freezes after DA starts, turn

off the computer for 30 seconds and then restart (It takes a little while

for the computer power supply to drain completely after it is shut off.  By

waiting long enough, everything should have a chance to reset.)  If the

program freezes at startup when you're trying various addresses, do not warm

start, you need to cold start your computer to reset whatever hardware was at

the address you tried.

Once the program is operating at the correct address, there should be no

further problem.  Just add the correct setting to your AUTOEXEC.BAT file

so it is set each time you use your computer.  You can type it in each time

if you want, but it is easier to use EDLIN or EDIT (DOS 5.0) to add it at

the start of the file right before or after the PATH or PROMPT statements.

In some rare events, you may obtain a DOS message "Out of environment space"

which means that you have lots of stuff in your PATH or other statements and

need to either remove some junk or increase the space by adding or changing a

line in your CONFIG.SYS file.  You can see what is hogging the environmental

space by entering SET without any parameters.  Things can be removed by using

SET itemname=   and pressing enter after the equal sign.

Example:  SET AVIX=

Refer to the SHELL command in your DOS manual.

EXAMPLE OF SHELL LINE IN CONFIG.SYS FILE

SHELL=C:\COMMAND.COM /e:256 /p

The example above sets the environment space to 256 bytes.

In the example below it sets it to 512 bytes.

SHELL=C:\COMMAND.COM /e:512 /p

(from page 566-7 in our DOS 5.0 manual)

OPERATION

DA-3 will automatically begin playing the disc from start to end shortly

after it starts as soon as the drive and disc are ready. The DA-3 Stop

comand Stops the drive, sets the drive sleep mode to 9 minutes, and exits

to DOS.  To stop play without exiting, use cue, pause or eject.

Quick Start

Start the program and play with it and various keys on the keyboard to

see what they do.  Most functions can be figured out without reading

further.  Press H for help during operation.  The Help screen will give

a short list of commands.  We have found that it works in a partial screen

Window with DOS using 256 Windows drivers, but the 16 color Win 3.1 driver

leaves the screen black for help.  Help works from DOS on the same display.

SCREEN DISPLAY

Adjust the CONTRAST and BRIGHTNESS controls on your monitor.  The program

uses black, grey, white, and bright white.  Some screen colors require that

grey be set to the correct brightness, or any 1/3 brightness color based on

grey will be black.

Play Time Scale

At the top of the screen below the program name display area is the play

time scale.  The scale is white and grey on black.  You should see the

scale across the full screen.  Adjust the brightness and contrast of your

monitor to display the entire scale, and lower the brightness so the black

display areas outside the scale and in the display time window just turn

from grey to black.  The time location of the laser is shown on the scale

by a diamond.  The diamond color is normally green.  Different colors are

used to show other play modes.  The scale is used for play position

indication and is used to directly set any play minute using the cursor left,

cursor right, and cursor down keys.  This works a little like the cueing

mechanism on a manual turntable for phonograph records (if you can remember

them).

The scale indicates from 0 minutes to 79 minutes.  If you have a 80 minute

CD, it will be off scale, but should still play, drive and disc tolerances

permitting.

Play Time Window

This window displays the current Track number, Index number, current play

Minute, current play Second.  Track numbers range from 1-99, Index numbers

from 0-99, (zero between tunes), Minutes from 0-80 depending on the length

of the disc, and Seconds from 0-59.  Frame numbers range from 0-74 and

they are only displayed in the section repeat window.  The time display

normally shows the AMIN and ASEC times, which show the total running time

location from the start of the program area of the disc.

You can toggle between running time on the disc and running time within the

track by pressing the G key.  G is for Get track time.

At the left of the Track digit display the track control information will be

shown if it is unusual.

We have found only a few discs which have Index numbers higher than 01.  The

Index number is 00 between the end of a one track and the start of the next

track.  A few discs will also let you see the track elapsed time count down

when the Index is zero, from 2 to 1 until the start of the track when it is

zero.

At the left of the track digit window, in the top position, a small green dot

is used to indicate Pre-emphisis on.  At the same location a star * is used

to indicate a CD-ROM data track.  A pair of lines is used to indicate reserved

mode 65 and a trio of lines is used to indicate reserved mode 192.  A dot is

used in the bottom position to indicate that the Digital Copy Permitted bit

is enabled.

Tracks, Minutes and Seconds Display

This shows the Maximum number of Tracks on the disc and the ending time of

the disc.  The remaining play time of the disc can be displayed at this

location by pressing the Y key.  Y is for time left Yet.  This is selectable

because a countdown display makes some people uneasy.

Drives Display

This shows the ID of the active drives accessed by the program.  The current

drive is indicated with a yellow block, active drives by green block.  Drives

not accessed yet or IDs which are unused are indicated with a - indication.

This drives display is active during drive selection.  Drives are selected by

Pressing D and pressing the number digit key at the prompt.  The program

searches through higher IDs if the selected ID does not respond, and stops

at the first ID where a drive responds.  A single drive should be Id#0.

Pressing D will Display the Drives.  Pressing Esc or spacebar at the drive

prompt will clear them.  Pressing S at the drive prompt will leave the icon

for the active drive on screen below the display.

Drive Icon Display

This program uses two icons. The CDR1700S icon is used for the 1503S and

the CDR3700 icon is used for the CDR3500 and newer drives.

The Hitachi model number is shown if the program knows the drive type.

Lights on the CDR1700S drive icon change with and without

a disc and during some conditions which require a seek.  The icons are

used for disc indication and are helpful if you have two different drives

on a computer.  See above for hiding the Icon.

Operating Key Display

The display for these keys changes when activated except for Stop (F2),

Begin (F11) and Finish (F12).

The play key is lit to indicate play.  Activating Pause by Pressing the

Spacebar will display a pause key.  This is normally hidden since the

same location is used for other functions and prompts.

The main operating keys are shown on screen and represent the function keys

F1-F12.  From left to right are:

KEYS        Function

F1 A E      Accept disc (close tray) or Eject Disc.

F2           Stop/Exit with 9 minute sleep reset

F3 P        Play - operation of the play key depends on the mode.  In

the normal mode, starts play from the beginning of the disc.

In Track Repeat, plays from the Starting Track.  In Section

Repeat, starts play at the preset Begin Time.  In section

repeat mode, can be used to restart play at the beginning

of the marked section.

F4 <        Skip backward one track.  Restarts current track when the time

within the track is more than a few seconds.  Wraps to last

track if playing the first track.

F5          Skip backward one minute.  Wraps to last minute if playing

minute zero.

F6          Skip backward 10 seconds.  With wrap to last few seconds.

F7          Skip forward 10 seconds.

F8          Skip forward one minute.

F9 >        Skip forward one track.

F10  L      Left/Right/Stereo channel selection

Used in channel identification and in playing CD-ROM audio tracks

which have different audio on the left and right channels.

The key lights red in S L R modes.

F11 B       Begin mark for section repeat. Start track during track repeat.

F12 F       Finish mark for section repeat. Finish track during track repeat.

The F11 and F12 keys only operate on AT class machines.  This

is apparently due to the lack of BIOS support on XT class

computers for these keys.  These keys can be read by other

means but the results are inconsistant between computers.

On an XT use the keyboard keys.

1-9  N      Number play - play by starting and ending track number

The number keys (1-9 & 0) are used to enter the numbers during

Number play, Repeat play by track, to insert or delete a

track from the Que and to set the drive ID number.  A starting

track number sequence can be started simply by pressing the digit

keys 1 though 9.  If the disc has 9 tracks or less, the single

digit is accepted.  If the disc has 14 tracks, and the first

digit is 2, track 2 is accepted as the first track since tracks

20-29 don't exist.  Pressing Enter or the spacebar will accept

single digit track numbers or use the defaults for starting and

ending track numbers when no digit is entered.

Q           Quit the program with drive locked

ESC         Escape from some prompts.  Escape from program with drive

unlocked when not in a prompted entry mode.

U           Unlock drive. Enable drive Eject button, which is normally

disabled when a disc is accepted.  Eject is normally disabled

when the program is exited so play isn't accidently terminated

by pressing the eject button.  Pressing the wrong button is

easy on the 3600 & 3700 drives.  Drive locking keeps nosy

people from messing with your discs or drive, but there are

times you would like to leave the drive unlocked without

stopping play.

SlideCue Operation

The cursor Left, Right, Up and Down keys are used to Cue and play to

any even minute on the disc.  Very useful when the disc has long tracks,

and when setting section play time or sampling the content of tracks on

a new CD.  The Left and Right keys move the location.  Cursor up cues and

reads the current track code at the new location.  Cursor down starts play

at the location.  Much easier than typing the exact play time in.  Used with

the 10 second backward and forward keys to find the exact location.

Cue Current Track Start

Pressing C will Cue to the start of the current track and set pause.  Pause

can be released with Cue or with the Spacebar.

Walk Through Disc Tracks

Pressing W will play the first 17 seconds of the each track.  During play

you can press F9 to advance to the next track.  Any other key will exit the

disc walk through.  The Walk feature is used most often with a new CD.  After

playing the start of the last track, play begins at the start of the disc.

Que Play Mode Operation

Pressing the / key during normal operation starts a shuffled random play and

displays the next track number in the Que.  The diamond on the scale changes

color.

When the Que is active, the Play key will start the next track and the Que

rotate keys are active on the keyboard (unshifted < or >).

Pressing the T key Toggles the Que on or off.

Pressing the M key activates the Manage Que function.  This lets you

rotate the Que list with the < or > keys (shift not required),

Clear the Que, Insert or Delete, fill the Que, or shuffle the Que.

Pressing the spacebar exits the Manage mode.

During Que play, play from the Que will repeat until Que mode is cleared.

Repeat Functions

DA-3 will normally repeat a disc until it is changed or the program is

exited.  There are two other repeat modes which can be selected with

the R key.  This key is three way.  Track Repeat and display, Section

Repeat and display, and display off.

Track Repeat

The Begin Track and Finish Track can be set by entering the track numbers

with the keys or during play by pressing the B and F keys,  You can move

around the disc with SlideCue or Forward or Reverse Track.  Press the

Play key to start at the beginning track.

Section Repeat

During play or when the Begin and Finish times are displayed, Press the B

key to select the point to Begin a section repeat, and the F key to select

the point to Finish a section repeat.  Set section repeat active by with

the S key or with the R key.  When the Begin time is higher than the

Finish time, the Finish time is set to the end of the disc.  When the Finish

time is set lower than the Begin time, the Begin time is cleared to the

start of the disc. The Begin and Finish times cannot be set closer than

one second.  The AMIN ASEC and AFRAME times are used for section repeat.

The Begin and Finish times can be set on the fly by holding the key down.

The main display will not be updated until the key is released.

To play from the Begin point to the Finish point, check the color of the

laser position indication to verify that Section Repeat is set, then press

the play key.  If Section Repeat is off and play starts at the beginning of

the disc, just turn it on with the S key, and press play again.  Section

repeat will exit section repeat mode after 30 repeats.  Very tight section

repeats would prevent exiting with a keypress on some computers so the

begin and finish of a repeat was set to 1 second minimum and 30 repeats

maximum.  If needed, toggle section repeat back on and press play again

to repeat another 30 times.

When the Repeat window is off, section play can be activated with the S key.

The S key has three modes which advance with each key press.  On with the

Section Repeat Begin minute and Finish Minute shown above the play time

scale, On with the Section play scale hidden, and off.

This feature is useful when making a play list for programmed play of an

audio disc for instruction purposes or in learning the words to a song.  An

advanced version of DA3 may make a play file using this feature in

combination with a text file.  To use these times directly, use the

command line player DA.EXE with play times in a batch file.

Color Adjustment

You can select the screen color while using the program if you have a VGA or

EGA color monitor.  The default colors for CGA are Bright White Ink, and Blue

Paper.

The default colors for EGA/VGA are light blue paper (P17) and Bright White

ink (I63).  The Z and X keys rotate the colors from the current

settings through all 64 colors (0-63).  Color 0 is a black screen, color

63 is bright white. Pressing V sets the Default colors to the setting

established with the SET AVIX=xxx Pxx Ixx statement or to the program

default setting if the AVIX variable does not include color settings.  There

are several screen colors which work well and some really ugly combinations

with Ink colors other than I63.  Determine the color number by rotating

the colors in either direction from full white (63) or full black (0).

The color settings are not available when you run DA3 in a partial screen

window under windows 3.x or a DOS window of OS/2.  The appearance of part of

the display is altered and the drive icons are flattened.

If DA3 is left active for several days with very bright colors, part of

the screen color may become weaker.  Reduce the brightness of your monitor if

you leave DA3 running overnight or for several days in a row, or change the

paper color from blues to greens or reds every few hours or days.  This is

not a problem with DA3 but can happen when any constant image is displayed

for a long time.  A screen save color rotation mode may be added in the

future for EGA/VGA hardware.

Default Startup Drive

You may want to start up with a default drive other than ID#0.  Add the

Dx option to the AVIX= line.  For ID#1 as a default, SET AVIX=300 D1 P49 I63

Detection of Maximum Drives

The CDIFI8A and CDIFI8S interface cards can support up to eight drives.  We

check the card and verify that the card has the identification bits set and

that it mirrors the drive select bits at the check port.  If you have a drive

attached to the card which has only 4 id settings, drive IDs from 4-7 will

be drives 0-3.  Drive 0 will respond at 0 and 4, drive 1, at 1 and 5, drive

2, at 2 and 6 and drive 3, at 3 and 7.  If any 4 ID drive is used on an 8

drive card, or the drive housing wiring or cabling has a one of the control

lines for drives 4-7, grounded or missing a wire, the higher drives

cannot be accessed with either this program or any other.  If you have

two interface cards in your system, you can access the drives on one or the

other.  To change cards, change the AVIX=xxx address used.  You may want to

make a batch file for each address setting.

Laser Operation Check

The 1600S, 1700S, 1900S, 3600, 3700 and 6700 drives have a check mode which

allows reading low reflected laser light when a disc is installed.  This

check is supposed to be used with a disc installed in the drive and with the

drive in pause.  It seems to work ok during play but takes longer.  This

program checks it 80 times and shows a display above the time scale.

A straight line means the check was ok.  A fuzzy character means low light

caused by dust or whatever.  A block means the laser was turned off during

that part of the check.  With a disc in the drive, press the O key.  This

is the O for Operation check, not a zero.  To see the Laser stopped,

unlock the disc, press O and press the eject button on the drive.  Selecting

this function on drive types below type 5 should result in a beep.  Drive

types above 9 don't exist yet.  This function will try to access them when

they exist, so if turns out that they are not compatible...

Utility Functions

Pressing I shows the drive Identification string and copyright notice.

The program uses this information to determine what drive type is active.

Pressing V shows the program version number, program size and date.  The size

should match the file size unless a modem program added bytes to the

end of the file during transfer or some virus has attached itself to the

end of the program.  Suspect something is wrong if they don't match.  The

V key also resets the virtual paper and ink colors to the default presets.

Reset

The program will try to Reset the drive if it is a CDR1503S and it cannot

obtain focus on the disc, or a CDR3600 which does not start play at the

beginning of the disc when instructed to do so.  We allow 10 seconds before

trying to access the drive after Reset to give it time to focus, re-read the

TOC and adjust its track pitch settings.  Reset can be manually engaged with

the ! key (shift+1).  This operation is alnost never needed but it is

available as a alternative to a power-off for 30 seconds cold start.

The # key will force a re-read of the TOC maximum track number in case it

fails the first time.  This function is left over from debugging, if the

track number is different after you use this key, you may want to eject the

disc and re-insert it.  (# is shift+3)

Operation With Other CD-ROM programs

Audio player programs which stay resident will cause misoperation of DA3 if

they are active.  Notable are the "Hitachi CD" player, the Software Toolworks

Jukebox and the old Amdek player.  If you want to play CDs on a higher ID

while using a single CD in the lowest drive ID, start play with DA3 and exit.

The other program will need to use the card and drive bus.  Do not try

loading two copies of DA3 under windows to access two different drives on

the same interface card if both will be active at once or either will be set

to background operation.  A similar caution applies to OS/2.

Access to two drives is possible with two programs only when two different

interface cards are in use with completely independent access software.  For

example, this program on one card and the extensions and driver on the other.

Under Windows 3.1, DA3 can be used with the exclusive and/or background

options set.  If the background mode is set, it will operate with reduced

time slicing, and will slow down other tasks.  If background is not selected,

it will only access the drive and update the display when it is the active

window.  The options can be changed by selecting the upper left window button

and options.

You can setup DA3 under windows by making a PIF file which loads it in a

window, and set it up with the CD icon with properties.  Then you can open

both windows and drag it to accessories or whatever, and delete the original

window used to set it up.  See Windows Help for PIF.

One copy of DA3 can be used in the forground or background.  For operation

in a partial window from the file menu, load DA3 with the minimized option

set.  Click on the icon and set the options for background and partial

window before selecting Switch To.  DA will begin operating in the background

almost immediately.  Set one of the repeat or ShufflePlay modes and then do

your thing with something other than CD-ROM drives under windows.

Defect Sensing

DA3 uses the system timer to set the duration of its response loops to

about 4 seconds.  This is shortened during drive ID checks.  If the drive

fails to respond within the time, DA3 times out and exits to the timeout

prompt allowing you to escape or retry.  Timeout is checked at the start

of each command.  If the drive is turned off at the wrong time when the

program is running, it will take 4 seconds for each byte requested from the

drive.  This could be 40 seconds during a timecode read or much longer when

the drive is first selected.  The MS-DOS CD-ROM extensions timeouts are much

too long to allow the free selection and detection of drives used by DA3.

The Extensions driver timeouts are very long.  Try to do a DIR on a the

CD-ROM drive with an audio disc in the drive or clicking on the CD drive Icon

under windows, or some kind of DOS shell program and see.  The only way to

recover is to put a CD-ROM disc in the drive when it eventually times out.

To prevent this problem with the extensions, you may chose to put the MSCDEX

line in a separate batch file and remove it from your AUTOEXEC.BAT file.

Then you can load the extensions with the batch file only when you want to

use a CD-ROM disc.

If you find any bugs with DA3, please report them with enough info to help

us duplicate the problem.  We believe that the program properly detects

a drive problem with failure to report no disc installed after startup

on the 3500/3600 type drives.  There may a possible problem with the drive

sleep mode on 1503 or 3500 type drives.  The newer drives don't sleep until

a audio disc finishes playing.  The 1503s drive does not have a separate

channel command. So when the channels are selected, the current time is

obtained and used as a start time after setting the channel mute bits.

The Help screen uses page flipping to avoid disturbing the normal display.

It should appear in in the default colors on CGA, EGA or VGA.  If the

screen is black after pressing H or returning, please let us know, and

avoid pressing H.  We once saw a problem under windows where the text area

was colored but the remainder of the screen was black. If this is a problem,

the next version will need to rewrite the entire screen for help and normal.

These programs work in OS/2 2.0 in a dos window or shell.  To use DA3 at

an address other than the address found by autoscan, make a PLAY.BAT file:

SET AVIX=360

DA3.EXE

We have separate command line players and locking software in distribution.

These have been updated to use the AVIX variable for the address.

DA.EXE (version 3.0 and 3.1) and Romlock.EXE which now supports a

variable number of drives, a software unlock key and AVIX=xxx addressing.

Changes in this version fix wrong model display for 1503S(Sep93 version) and

single key Left/right/stereo toggle with section repeat function for F11 &

F12.  Help added in response to user request.

This software has been a labor of love which represents man-months of program

development time and signicant costs.  The initial versions of DA3 span the

years from December 1987 to now, several programming languages and two very

different computers.

We hope you enjoy using our software and appreciate the imagination and

care which has gone into the program design and operation.

AVIXIVA Reflective Systems

Lynwood, California

<end of file>