-----------------------------------------------------------------------------

PRC - Diagnostic program for Play/R and Play/D

Copyright 1993 FM Software, All rights reserved

-----------------------------------------------------------------------------

PRC is intended to exercise various MIDI engine API functions by sending

commands to the TSR from the command line.  It may also be used to

control some useful player functions from languages which do not provide

an interrupt facility, but which do provide a way to execute DOS commands.

Run playd or playr first, then type prc and an API function number.

Respond to any prompts (for functions 4 and 8, the file name may be

added to the command line).  Where a device number is requested, 1 is a

appropriate response.  To test MIDI output, either load a file with

function 4, or use function 21 to play a single note.  To test input,

send some input from a keyboard or other device and then use function 24

to read it.  To test large dumps, you must have used -Sn on the playd

line to set it's input buffer to nK bytes.  Otherwise, polled input and

a small buffer are used.

API Function   Use

0         Pause play

1         Continue play

2         n/a

3         Rewind

4         Load file & start

5         Get song status

6         Get file load status

7         n/a

8         Load file & pause

9         Silence playing notes

10        n/a

11        Volume adjust

12        Reset MIDI device

13        Set timer mode

14        Get channel map

15        Set channel map

16-19     n/a

20        Get device name

21        Play a note

22        n/a

23        Check for waiting input

24        Get waiting input

25        Get millisecond time

99        Remove Play/D from memory