FSS (Flight Simulation Service Station)

(c) 1993 Roger L. Flynn

Beta Version 1.0

Nov 14, 1993

Introduction:

1.0  FSS is a Windows/DOS based program that creates

adventure scenarios which add ATC control to  Flight Simulator

4.0.  The idea is simple.  You visit the Flight Service Station

(FSS) to create a flight plan which, in turn, is input to the

adventure creation process.  At the FSS you enter a simplified

flight plan, including weather briefing, and detailed

approach/landing information.  This information is then used to

create an adventure (.ADV) file that provides complete Air

Traffic Control and weather generation for your flight, from

clearance delivery to touch down.  Each flight is a 'check ride'

and is scored by a check-ride pilot.  FSS provides ATC control

for vector approaches, DME Arc approaches, procedure turn

approaches, and simple approaches where you are cleared direct

to the Initial Approach Fix.  Landings can be ILS, VOR, VOR-A,

VOR/DME, NDB, LOCALIZER, and LOCALIZER

BACKCOURSE. Other features are randomly generated 'holds',

and randomly varied weather.

1.1  This is beta release software.  I am not representing it as a

finished product. FSS still contains some debug code and many

many rough edges.  I still find something that doesn't work right

with almost every scenario, and I am sure you will also.  The

download size is almost 1MB.  FSS.ZIP will contain the

complete current version.  Updates will be uploaded as

FSS99.ZIP where 99 is the most recent update version.  Updates

will contain only certain executables and documentation, and so

will be much smaller.  If this sounds like your cup of tea and you

would like to help me test it and refine it, I think we can all have

a lot of fun.

1.2  Hardware Requirements:

--------------------------------

At least a 386 with 4MB and about 2MB free disk space.

1.3  Software Requirements:

-------------------------------

DOS, Windows 3.1, Flight Simulator 4.0b (Microsoft)

USA East (SubLogic)

Aircraft and Adventure Creator (Mallard)

Low altitude enroute charts or Sectionals and approach plates for

the area you wish to fly.

-------------------------------------------------------------------------------

2.0  Installation:

----------------

2.1.  Create a directory named FSS.

2.2.  Download FSS.ZIP to this directory and unzip it.

2.3.  Using the text editor of your choice change the PATHS file

under FSS to point to the location of FSS required files.

Line 1:   EAST.SMS database.

Line 2:   Location of your FSS database (FSS.DB)

Line 3:   Directory where you want your adventures to go.

(Notice the required terminating slash)

2.4.  Using Windows program manager, create a Program Group

named FSS.

2.5.  Add a program item named FSS Work Sheet.

Command Line   c:\fss\vision.exe

(change the drive letter if necessary)

Working Directory   c:\fss

2.6.  Add a program item named FSS Create.

Command Line   c:\fss\fss.exe

(change the drive letter if necessary)

Working Directory   c:\fss

2.7.  Copy AAF.EXE to the FSS directory.

2.8. Change you AUTOEXEC.BAT file to load SHARE.EXE.

(required for the flight database.)  You can usually load

SHARE high so precious low memory is not wasted.

2.9  Copy hash.voc to your FS4 directory if you don't already

have it.

This completes the installation process.

3.0  Now we are ready to work through a few sample flights.

Double-click on the FSS Worksheet Icon.  This will load the

Worksheet program.  You will see the Borland ObjectVision

menu. ObjectVision is a Windows application so the interface

should be familiar to you.  Click on File as you would for any

Windows application.  Then click on 'open'.  You will see fss.ovd

under 'files'.  Double-click on fss.ovd to execute the FSS

Worksheet program.  The last active worksheet will be displayed.

3.1  FSS is distributed with some sample flights.  Let's look at

the first flight and go through the worksheet field by field.  At

the top of the form you will see some buttons.  You can click on

these buttons to perform database task.

TOP       Positions you at the top of  the database.

NEXT      Moves to the next record.

PREVIEW   Backs up to the previous record.

DELETE    Deletes the displayed flight.  Use with caution!

BOTTOM    Positions you to the last flight on the database.

CLEAR     Clears the form.  Does not delete a flight.

Next to the CHECKPOINTS heading are three more buttons.

UPDATE    Updates a flight after you have made changes.

STORE     Stores a new flight in the database.

INSERT    Inserts a new flight in the database.

General Data Entry Techniques:

------------------------------------------

4.0  You can move from field to field with the TAB key or by a

mouse click on a field.  You can use the bar at the side of the

screen or the page up/down.  To update a field, you need to

delete the current contents with the backspace key. This version

provides no data integrity checking at input time.  I will add

better editing in a latter release.

4.1  To enter a flight in the worksheet you should first decide on

the departure and destination airports, then study enroute charts

and decide on the best routing.  You then study approach plates

for the destination airport and choose a published approach.

Next is the weather briefing.  You are then ready to enter the

flight plan.  But for now, instead of entering a new flight plan,

we will study some of the supplied flights.  You will need the

low-altitude enroute charts L-23 and L-24, along with approach

plates NE-2 for New York and Pennsylvania.  These are readily

available from your local FBO or mail-order from Sporty's.

4.2  When you execute FSS.OVD, you will see the FSS

worksheet screen. Since the flight iptmdt was the last active flight

it is now displayed.  This is a flight  from

Williamsport/Lycoming Co. airport in north-central Pennsylvania

down to the state capital, Harrisburg.  The flight number is very

important in that this will be the name used for all files related to

the flight.  The adventure creation process will use this name and

add a '.TXT' and '.ADV' suffix.  I use the three-character airport

code of the departure and destination airport.  If  I have more

than one flight between the same airports, I add a number to the

name.  For example, iptmdt01, iptmdt02, etc.  You can use

whatever convention you like as long as the flight number is no

longer than eight characters.   When you enter a flight number, a

search is made of the flight database and if a matching flight is

found, it is displayed.  If not, the input screen is cleared and you

are ready to enter the new flight.  In our example we see flight

iptmdt.

4.3  In the beta version, Pilots name is for documentation only

and will not appear in the created adventure file.

4.4  Aircraft type is actually comprised of two parts.  The first

should be the maker followed by the aircraft type.   For example,

Beech Bonanza, Cessna Skylane, Piper Aztec.  In ATC

communications the first word will be used along with the ID.

For example, "Cessna N8610R, you're cleared for the ILS RWY

13 approach, maintain 2200."  You can also use an airline name

and flight number instead of aircraft and ID.  If  you enter

"Delta" for aircraft type and "210" for the ID, your ATC

communications would read like this,  "Delta 210, approaching

Johnstown VOR, descend and maintain 4000".  Currently, the

program is not aware of aircraft performance.  It does not

distinguish between prop and turbine aircraft.  I hope the ATC

control will be generic enough to work with a wide range of

aircraft.   I have tested it with various types and it seems to do

OK.   In a later update, I will add weighting factors for larger

aircraft.  Most testing was done with single and twin prop

aircraft.

4.5  Time and Date are for documentation purposes only and not

currently used.  Maybe in a later version I will use the date to

generate seasonal weather and set the time in the adventure..

4.6  We have already discussed the ID field.  You can use

whatever you want here but whatever you enter  will appear in

all ATC communication directed at your aircraft.

4.7  Departure, Destination, and Alternate ID's must be in caps

just like they appear on approach plates or charts.  The Alternate

ID is optional, but of course, the Departure and Destination ID's

are required.  I have yet to find an airport that is not in the USA

East database, so you should be able to fly just about anywhere.

4.8  Comments are just that.  Enter whatever pleases you.

4.9  You can make the weather briefing as complicated or as

simple as you want to.  The three levels correspond roughly to

FS4 levels.  Departure weather will be in effect up to the first

checkpoint, where it will change to enroute.  Enroute weather

will be in effect till approximately the Initial Approach fix, where

Destination weather will kick in.  Notice that there are no

separate levels for winds aloft and clouds.  It was just too much

data to deal with.  So here is how it works.  Cloud levels are as

specified.  Winds for level one extend from surface up to the top

of level one.  Level two winds extend from the top of  level one

to the top of level two.  Likewise, level three extends from the

top of  level two to the top of level three.

4.10  Cloud levels one and two are for normal clouds.  Cover

(CVR) is a number 0-8 per normal FS4 inputs.  I use only 0 or 8,

clear or overcast.  When AAF for FS5 comes we can probably

use other values, but the ping-pong ball clouds of FS4 just won't

do.  Level three clouds are thunderstorms.  As with normal FS4

thunderstorms, the height controls intensity, while Cvr controls

storm density.  Again, you can be as detailed as you wish.

Whatever you specify will be varied by a random factor.  Also,

turbulence values will be generated randomly, so don't be

surprised when you encounter clear air turbulence.  In our first

example (iptmdt), we depart with clear skies which continue

enroute till we approach the Susquehanna river were we start

picking up some overcast.  When we make our final approach,

we will be close to minimums.

4.11  Next we will decide on routing.  You will always be

'cleared as filed', unlike real-world. Looking at the enroute charts

we see that the most direct routing via Victor Airways is V31

down to Selingsgrove VOR then to Harrisburg VOR, also via

V31.  We will intercept the airway at WATSO intersection, then

to SEG VOR, then on down to Harrisburg with a checkpoint at

MORTO intersection.  Harrisburg is our Initial Approach Fix.

Now back to the Worksheet, we next see navaid type.  Type is

required to find the navaid on the USA East database.  Valid

types are:

A    Airport

F    Fix or intersection

I    ILS's (Localizers,etc)

N    NDB (LOM, Low altitude, and High altitude)

V    All types of  VOR's

4.11  Next is the Navaid ID.  This field is case sensitive, ie you

must enter it exactly like it appears on charts.  Normally, navaids

are entered in caps with the following exception.  For NDB's you

should enter the first letters in caps and as much of the name as

possible in lower case letters.  Notice, for example, that to

specify  the FAF we enter 'N' to indicate an LOM followed by

'MD En' to identify the ENOLA LOM.  VOR's and airports are

three characters while intersections are five.

4.13  Next, you enter the altitude you want to be cleared for.

This should be somewhat greater than the Minimum Enroute

Altitude for the segment.  You can enter the MEA if you wish

but it is not used in this beta version.  Later, a check will be

added, and if you descend below the MEA you just might fly

into an obstruction.

4.14  Next field is the route you will be flying.  There are two

choices: direct or via a Victor Airway.  To specify a direct

routing, just enter DIR in this field.  You will then be cleared

direct to the navaid.  Normally you will file via Victor airway

routing.  Since the EAST database knows nothing of the airway

system, you have to enter one other piece of information: the

radial that defines the airway.  The specified radial is the radial

(NOT COURSE) of the navaid that you are flying towards.  If

this is not accurate, you will received some  very confusing

directions from ATC.   In our example, V31 is defined by radial

17 of the Selingsgrove VOR (SEG).  The next leg is defined by

radial 26 of the Harrisburg VOR (HAR).  That's all that is

required to enter a flight segment.  You can enter up to 10

enroute fixes.

4.15  After completing the enroute phase of flight planning, you

should then study the approach plates and decide on an approach

and published instrument landing.  FSS supports the following

types of approaches and landings:

APPROACHES

---------------------

Vectored Approach

ATC will provide vectors to intercept the final approach.

Procedure Turn

You will fly to the IAF then outbound, execute a procedure turn,

then intercept the final approach course inbound.

DME Arc

You will be vectored to intercept a DME arc, then fly the arc to

intercept the final approach course.

Direct to Initial Approach Fix

This is the default if none of the other approaches are specified.

Normally used when the termination of your enroute phase

roughly aligns you with the final approach course.

LANDINGS

ILS

Standard Instrument landing.  You are tracked down the

glideslope and localizer.

LOC and LOC/BC

Localizer and localizer backcourse.

VOR,  VOR-A, and  VOR/DME

Various VOR landings

NDB

Various NDB landings

The type of landing must be entered just as it appears on the

approach plate. For example:

ILS RWY 13

VOR RWY 7

NDB RWY 26

VOR-A

VOR/DME  RWY 31L

LOC BC RWY 6

Proper ATC directions will be generated for these types of

approaches and landings.

4.16  So now we enter the type of approach and landing on the

worksheet.  In our example, we are making a simple Direct to

IAF approach and an ILS landing to runway 13 at Harrisburg

International.  Study of the approach plate indicates a simple ILS.

The approach is via the IAF, Harrisburg VOR about 15 miles

from touchdown.  There are no step-down fixes, so we proceed

to the FAF while descending from 2800 to 2500 ft.  Notice that

we must specify the approach heading for each landing phase.

This allows for approaches that deviate from the standard

straight-in approach.  Decision height is entered directly from the

approach plate.  If you descend below DH while still in clouds,

disaster will generally ensue.

4.17  Next, you specify the Missed Approach.  This should be

entered just as it appears on the approach plate.  If you declare a

missed approached, you will be requested to fly this pattern and

will then be cleared for another landing.

4.18  This completes the flight planning phase.  You should now

click on the STORE or INSERT button for a new flight, or the

UPDATE button if you have modified an existing flight.

ADVENTURE CREATION:

5.0  Now that we have filed our flight plan with the FSS, we

proceed to the next step, which is to actually create the adventure

file.  The FSS program is currently a DOS based program

utilizing a simple menu interface.  (I will convert it to Windows

in a future release).  You should double-click on the FSS Create

icon to execute the FSS program.  You will see a menu with the

following choices:

a. Build Adventure

This is how you create an adventure from the FSS Database.

You will be asked to enter a flight number and will then see the

adventure creation process.

b. Display Airport

A quick and dirty display of airport data.  You enter the three

character airport id.  (Notice data enclosed in <>.  Haven't

figured this stuff out yet)

c. Display Navaid.

A quick and dirty navaid display.  You enter the navaid type (V

for VOR, N for NDB, and F for fix or intersection followed by

the ID).

d. Display Flight Plan.

Used for debug purposes.  Will be either fixed or removed from

the production version.

e. Show distance between two points.

Used for debug purposes.  Will be either fixed or removed from

the production version.

f. Show NE coordinates of a Navaid.

Used for debug purposes.  Will be either fixed or removed from

the production version.

g. Display Intersection.

Obsolete.  Will be removed from the production version.

h. Time Enroute.

Primitive. Will be fixed or removed from the production version.

i. Descend Distance.

Primitive.  Will be fixed or removed from the production version.

j. Descend Rate.

Primitive.  Will be fixed or removed from the production version.

k. Find any Navaid.

Very useful.  You enter one or more characters of a Navaid id.

You will then see a display of all navaids that match the input

string.  Can be used to find airports, VOR's, NDB's, Fixes, or

Intersections.

q. Quit.

5.1  Probably the only choices you will use much will be Create

adventure and Display any navaid.  So lets look at how we use

these two functions.

5.2  Let's step through the creation process.  We execute the FSS

program by double-clicking on the FSS Create icon.  We then see

the FSS menu.  We choose selection 'a', Build Adventure.  FSS

prompts us to enter a flight number.  We enter 'iptmdt'.  (lower

case)  FSS then displays the flight number, pilot, aircraft type

and aircraft ID.  We then see a display indicating success or

failure of  USA East database searches.  Each Navaid specified

on our worksheet must be found on the database or we will get

an error.  In our example we see "Looking for f-WATSO",

followed by the NE coordinates and a check mark.  This

indicates that the Navaid was found.  The 'f' indicates a fix or

intersection.  If a navaid is not found, you will see 'not found'.  If

this happens you should consult your charts and use 'Find any

Navaid' to locate the proper ID.  You would then execute FSS

Worksheet and make corrections.   But for our example, we see

that all the navaids were found.

5.3  You should then see "Building IAF Approach Segment".

This indicates that we are creating an Approach directly from the

IAF.  At this point, an Adventure source file is written to a file

named IPTMDT.TXT.  Then FSS invokes the AAF program to

compile the source file and creates an adventure file to

IPTMDT.ADV.  You should see a display of  total length,

number of  labels, followed by "compilation successful".  You

then  'hit any key' to see the menu again.  Enter 'q' to exit FSS.

We have now created an adventure!

FLYING THE ADVENTURE

------------------------------

6.0  Now that we have created an adventure file (IPTMDT.ADV)

we are ready to fly our flight plan.  I'm sure all of you have

evolved your own method of managing adventure files so just use

your own procedures.  Of course, you have to copy the .ADV file

to your FS4 directory.  You then invoke FS4 and load the USA

East Scenery area.  Choose your aircraft and then load

IPTMDT.ADV from the Adventure menu.  You will be

positioned somewhere on the departure airport and should receive

a message from ATC to contact clearance delivery.  Let's stop

now and look at the pilot/ATC interface provide by FSS.

6.1  FSS controls your flight by dividing each flight into

segments.  You move from segment to segment by complying

with ATC directions.  During ground operations, you move to the

next segment by setting your COM radio to the proper frequency.

Enroute, approach, and landing segments are completed by flying

to the proper checkpoint.  Let's look at the key strokes you use to

communicate with ATC.

6.2  R Key.

The 'R' key is the repeat.  It causes the repeating of the last ATC

communications.  This key also affects the flight segment control.

It is very useful, but can also get you into trouble.  You MUST

wait for a response from ATC before hitting the key again.  And

remember that ATC gets pretty busy and can't always respond

promptly.  If you don't wait for the response, you can screw up

your flight segment progress and can back up to previous

segments with confusing results.  You can use the repeat key

whenever you wish, if you are careful to wait for a response

before hitting the key again.

6.3  / Key.

The '/' key request a report from your 'check pilot' of your current

score.  You always start with a perfect score of 100.  (The only

perfect flight is one that never departs)  You loose points by not

promptly complying with ATC instructions.  If you miss a check

point you loose 10 points.  Departing on the wrong runway will

cost you 20.  Failure to properly track the Glideslope and

Localizer during approach and landing will cost you points.  I

have never  made a perfect fight.

6.4  X Key.

This is the 'whoa' key.  It backs you up to a previous segment.

Can be used to cancel an erroneous report, ie. hitting the

"Report Key, <ENTER>. You shouldn't have to use this much.

6.5  Enter Key.

The Enter Key is used to manually report a checkpoint.  You

have to fly within about 2 Nautical miles of each checkpoint to

successful move to the next flight segment.  If you miss a

checkpoint, you can manually report it.  This will cost you

points.  If you erroneously report a checkpoint, you can cancel

the report using the 'X' key.  Also used to report 'holding' while

flying a missed approach.  Having to use this key usually

indicates a pilot in trouble.

6.6  A key.

The 'A' key is used to acknowledge an ATC message.  It is for

cosmetic purposed only and is not required.

6.7  \ key. (back slash)

The '\' key reports a missed approach.  You will be requested to

fly the missed approach segment, hold at the proper point and

then report back to ATC.  You will then be cleared for another

approach.

6.8  E key.

The 'E' key is used to declare and emergency.  You will be

cleared to the nearest airport to make an emergency landing.  In

later versions this will be enhanced to allow ATC to vector you

to the nearest airport, clear you for an approach, and track you

landing.

6.9  Now that you are familiar with the interface, we are ready

for a shake-down flight.  After loading the adventure file using

AAF menu, we should be sitting on a taxi way at Williamsport

Airport in north-central Pennsylvania.  You should receive the

following message: "Bonanza N8610R contact Williamsport

Clearance delivery....clearance delivery frequency is 122.45."

This indicates that the adventure was properly loaded and is

active.  Clearance Delivery will now wait for you to contact them

on the specified frequency.  At this point I set up my Nav radios

and set the OBS for the first checkpoint.  For this flight, I set

NAV1 to Selingsgrove VOR (SEG) frequency 110.4 and turn the

OBS to heading 196. I then set NAV2 to Milton VOR (MIP)

frequency 109.2, and the OBS to radial 288. (The charts indicate

odd numbered radials but, of course, we can't use odd radials in

FS4.) After your radios are set the way you want them, you can

now contact Clearance Delivery on 122.45.  I normally tune the

frequency as soon as it is issued or else I'll forget it.  You can

use the Repeat Key (R) if you forget the frequency.

6.10  Shortly after setting COM1 to 122.45 you should receive

the following communication from ATC:

"Bonanza N8610R cleared as filed to Harrisburg International

Climb and maintain 2500, expect 4000 10 minutes after

departure...

Contact Ground on 121.90 when ready to taxi."

6.11  We should be ready to set COM1 to this frequency.

Shortly, you should hear the message, "Bonanza N8610R,

Williamsport ground control....taxi to runway 27 and hold

short...Contact Tower on 119.10 when holding short."

6.12  You should now taxi to the proper runway and hold.  When

holding short, set COM1 to 119.10.  You will now hear,

"Bonanza N8610R, Williamsport tower.....You're cleared for

immediate takeoff....fly runway heading....climb and maintain

2500"

6.13  Taxi onto the runway, take off, fly runway heading up to

2500 then level off.  If you don't climb promptly to 2500 you

might hear a message asking you to expedite your climb.  Shortly

after reaching 2500 you will be passed off to Departure control

and cleared to the first checkpoint, WATSO intersection.  You

will be cleared direct.  Since we took off to the west, we will

make a left turn to about 148.  When cleared direct you can use

the Repeat Key (R) to get current heading to the checkpoint.

You will also receive an altitude assignment.  You should climb

promptly to you assigned altitude.  If not you will receive a

warning from Departure Control and will loose points.  Keep

using the Repeat Key to re-check you heading.  You should also

have your radios set for WATSO, so you know when you are

approaching the intersection.  When about 2 Nautical Miles from

WATSO you should hear the following message:

"Bonanza N8610R, approaching WATSO intersection, radar

contact, center....."....you're cleared to Selingsgrove VOR via

V31...climb and maintain 6000"

6.14  You should immediately start your climb and as you pass

the intersection, turn right to heading 197.  The NAV1 one

needle should be centered.  You are now required to follow the

V31 airway.  If you wander from the airway, ATC will remind

you to turn right or left to rejoin the centerline.  You also loose

points.  So now we are flying at 6000 and tracking the airway

down to SEG VOR.

6.15  Let's talk about Holds.  As in real-world flying, you may

be asked to hold at any fix along your route.  You might see a

message like "Bonanza N8610R, hold at Selingsgrove VOR on

V31, expect further clearance in 10 minutes."  You should enter a

holding pattern at the specified point and then hold until cleared

to continue.  You should not wander from your assigned altitude

or get more than 5NM from you hold point. You can cancel a

Hold at any time by hitting the Report Key (Enter).

6.16  Assuming, we don't have to hold we will continue on V31

to the SEG VOR, where we will be cleared by center to MORTO

intersection, also on V31.  At MORTO we will be handed off to

Harrisburg International Approach control and cleared to the IAF,

Harrisburg VOR.  You will hear the following communication:

"Bonanza N8610R, Harrisburg International approach control.

ILS RWY 13 approach in use. You're cleared direct to initial

approach fix, Harrisburg VOR.

Heading should be 206....

Descend and maintain 2800"

6.17  Turn to the specified heading and start your descent.

You're still about 30NM out.  When you arrive at the HAR VOR,

you will see the following:

"Bonanza N8610R, at the IAF, Harrisburg International Approach

control.  ILS RWY 13 approach in use. Descend and maintain

2800. Intercept final approach course of 126."

6.18  NAV1 should be set for the Localizer (110.9) and the ADF

should be set to the Enola LOM  Start a shallow turn to the left

to intercept your approach course.  Shortly, you will be instructed

to descend to 2500 when established on the Localizer.  When

you arrive at the final approach fix you will see, "Bonanza

N8610R at the FAF inbound, you're cleared to land runway 13".

Now  just track the Glideslope inbound to touch down.  Sounds

easy, huh?   If you descend below decision height while in

clouds you will probably hit an obstruction.

6.19  After touchdown you will be instructed to exit at the first

taxiway and will see your score.  This concludes the first flight.

SECOND FLIGHT (mdt22n)

7.0  Let's now look at another flight that will demonstrate some

quirks with the SMS database and with FSS.

7.1  We will now file a flight plan for a flight from Harrisburg

Int. up to Jake Arner Memorial in the vicinity of Allentown.  The

flight number is 'mdtn22', again, the ID's of departure and

destination airports.  Start up the Worksheet by double clicking

on the ICON.   You can use the 'next' and 'previous' buttons to

find 'mdtn22' or you can enter the flight number on the worksheet

which will retrieve the flight plan.

7.2  Notice that we have now set weather for Departure, Enroute,

and Destination.  We have specified levels one and two.  Surface

winds are from 270 at 8 while winds aloft are from 270 at 10.

There is a shallow layer of clouds at 2000 and another shallow

layer at 3000.

7.3  Our routing will be direct to BOBSS intersection to intercept

V162, then we intercept V93 at DUMMR and head up north to

Jake Arner.  We are cleared up to 5000.  Our Initial Approach

Fix is the Carbon NDB (LQX).  Notice that we enter 'LQX C' for

id.  Looking at the approach plate reveals a typical NDB

approach.  You fly to the NDB, turn outbound on the specified

heading, make a procedure turn, fly inbound descending to the

MDA.  Notice that there is no Final Approach Fix as such.  FSS

Procedure Turn approach requires an FAF.  So what we will do

is specify LQX as the IAF with distance specified as 1 mile.  We

then enter a fictional FAF at 4 miles from the airport.  FSS will

direct us to fly outbound on the reciprocal of the inbound course,

make a procedure turn, intercept the inbound course, then clear

us to land when we reach the fictional FAF.  (Most approaches

do have a published FAF so the fictional one is not necessary.)

7.4  It is very important to note how approach fixes are specified.

Basically, there are two types, Navaid fixes and distance fixes..

For fixes defined by a Navaid, you must enter the ID AND the

distance.  For fixes defined only by distance you must enter

ONLY the distance.    I usually round up to the nearest mile.

This allows for entering the IAF, FAF, and up to two step-down

fixes.  You should enter only one of the approach types (Vectors,

Procedure Turn, and DME arc).  If you don't specify one of the

above, a 'Direct to IAF approach will be created'.   You can

combine any approach type with any published landing, but some

combinations will make more sense than others.  You probably

would not want to specify a vector approach for the above NDB

landing.

7.5  Now that we have studied the charts, approach plate, and

Flight Worksheet, let's go fly.  We start the flight at Harrisburg

International and are cleared to Jake Arner via V162 and V93.

We will be cleared to depart on RWY 31 since the wind is from

the west.  We intercept V162 at BOBSS intersection.  Our

heading will be 80.  After flying along the airway for awhile, you

should get a warning from ATC the you are on the wrong radial

for V162.  I'm not sure why.  Maybe the ETX VOR location in

the SMS doesn't match real-world charts?  I just don't know.  But

its gonna cost you points.  I just keep flying based on my

instruments and ignore the warnings.

7.6  When you approach SNOWY intersection you will be passed

off to Jake Arnold approach control.  (All airports in FSS have

full ATC regardless how humble. We can work on this in future

releases).  You will be cleared to the IAF, LQX NDB.  You

should fly toward the NDB.  When a mile or so out, you will be

requested to fly outbound to the procedure turn.  So turn

outbound at station passage and start your timing.  No DME

here.  At about 7 miles from the airport we will be requested to

make a procedure turn to intercept the approach course inbound.

After making the turn, we fly inbound on 260 to the 'FAF' where

we will be cleared to land.  By the way, your score will be

displayed after touch down.  This concludes the second sample

flight, successfully I hope.

THIRD SAMPLE FLIGHT (22nlns)

8.0   For our next flight we will fly from Jake Arner to Lancaster

where we will make a Vectored approach to the ILS RWY 8

landing.  So, call up flight 22nlns, and let's look at the worksheet.

We have a couple of shallow cloud layers, again with surface

winds from 90 at 4 and winds aloft from 100 at 10.  We will

intercept V12 at the ETX VOR then proceed down to LNS via

V12.  We will be cleared up to 6000.

8.1  Notice that we specify 'Yes' for vectors, and request ILS

RWY 8. We will be vectored to intercept the final approach

course of 77.  Notice that the FAF is the outer marker.  The

MANOR OM is not in the SMS database so we have to specify

the distance from the runway, in this case 6 NM.  If you specify

a Navaid that is not on the SMS the adventure creation will fail.

Also notice that we must specify the distance from BOYER, our

IAF, to the runway.  This distance is used to transition from

enroute to the vectored approach.

8.2  About 15 NM from LAN VOR we will receive vectors to

intercept the final approach course.  We are approaching from the

east and will be landing back to the east (approach heading 77)

so we will be vectored around a rectangular course.  You will be

instructed to turn right or left to a certain heading.  You are

expected to fly that heading maintaining current altitude until

receiving the next heading.  You can use the Repeat Key (R) to

keep on track.  You will be instructed to intercept the final

approach course.  You should have VOR1 set for the ILS and as

the needle starts to center, turn inbound to the approach course.

You will then be cleared to the first fix (in this case the FAF)

and assigned an approach altitude.  At the FAF you will be

cleared to land.  You just track the ILS inbound to touchdown.

FOURTH SAMPLE FLIGHT (lnsjst)

--------------------------------------

9.0  This flight will be from Lancaster over to Johnstown via

V474 then northwest on V469 to the DME arc approach into

Johnstown-Cambria Co. airport.  We will be making the

VOR/DME RWY 23 approach and landing.  During the initial

segments we will be flying direct from fix to fix, until reaching

SCAPE intersection, where we will fly V469.  At St. Thomas

VOR (THS) we will pick up V469 up to TATES intersection

where we will be cleared to the DME ARC approach.

9.1 Looking at the worksheet, we see that to specify a DME arc

we enter the ARC ID and distance.  That's all it takes.  When

cleared to the DME arc approach, you will be expected to fly

towards the airport, intercept the arc, fly the arc until intercepting

your final approach course.  You then track inbound to the FAF

and will be cleared to land.  In this case we are making a

VOR/DME approach.  DME because DME is required to locate

the FAF.

CONCLUSION:

------------------

10.0  As stated earlier, this program is still under development.  I

have been developing and testing it for almost a year.  Much

remains to be done.  There are numerous problems.  But I think

it is at point where I need additional help testing.  Most flights

should be at least flyable.  You might have to ignore some

warnings and you may loose points when you haven't done

anything wrong.  Some events are timed and the timing depends

on many factors, scenery density, CPU and Video performance,

aircraft performance, etc.  I will try to fix bugs and design

problems as they are reported.  But please be patient.  This is,

after all, a spare time project.  And spare time is hard to come

by.  I hope you enjoy FSS as much as I have enjoyed working on

it this past year.

--------------------------------------------------------------------------------