CDDA Version 1.0a - This is the second public release of CDDA.

CDDA - What is it?

------------------

In February 1993, I needed to get some audio samples into digital

form on my PC.  Toshiba had just released the 3401 CDROM drive

which allowed reading of DA frames across the SCSI bus.  Because

there were no programs available to do this, I had to write my

own.  I used it to get the samples, and then left it alone because

I had no idea anyone else wanted to do this.  Some months later

CDGRAB came out, but they wanted much too much money for their

program, so I cleaned up the program and released it as CDDA09A.ZIP.

Note:

Some time later I heard from a very irate person who told me I

couldn't use the name CDDAxxx.zip because he was already using

the name.  I've looked everywhere, used ARCHIE, and asked people

who seem to know every program on the net, and no one has seen

this guy's program.  However, in the interest of keeping peace on

the net, I have changed the distribution file name to DA2WAV1A.ZIP.

The program itself will still be called CDDA.

The first release fully supported the Toshiba 3401 CD-ROM drive.  I have

added the Sony 561/8003, the NEC 84/74/3X? and MSCDEX only to this release.

Unfortunately I don't have a Sony/NEC or drivers that allow the MSCDEX

part to work.  I have given prerelease versions of my program to people

who have to some degree had success with other drives.  I have no

guarantees that this program will work with your drive.

Different ROM revisions cause the drives to do things differently.  One

version may work just fine and others might not.  It is very tough for

me to help with drives that don't work, when I don't have one of

those drives to work with.

Because the Toshiba/Sony/NEC drives need proprietary SCSI commands to

read the audio sectors, I chose to talk to the drive through the ASPI

interface.  I hope that most people with SCSI drives have gone the

ASPI route.

What drives support DA?

-----------------------

I have heard from many different people who say such-and-such drive

will read DA frames.  The following is a list of which drives which

I understand are supposed to work: (thanks to bwilliam@iat.holonet.net

for starting this list)

Apple 300, 300i (Sony CDU-8003, CDU-8003A)

Apple CD300+

Chinon CDS-535

NEC CDR 300, 400, 500, 600 series

NEC CDR 84-1/74-1 (with special firmware)

Panasonic CR-562B/563B

Sony CDU-561

Toshiba XM3301 (Silicon Graphics)

Toshiba XM3401

Toshiba XM4101

These are rumors that one of the Mitsumi drives will work using

the MSCDEX interface.

How do I use this program?

--------------------------

This is a command line utility which allows the user to specify the start

and end points of the data transfer and the output type.  The start and

end points may be entered in one of three modes,: LBA, MSF and track.

Logical Block Address mode is the number of the frame from the start of

the disc. Minute, Second, Frame mode specifies the time from the start

of the disc in actual time.  Track mode allows you to dump an entire track

(or song) to disk.  The LBA and MSF are related by the following formula:

LBA = Minutes * 60 * 75 + Seconds * 75 + Frames - 150

The lead in track is usually 150 because there are usually 2 empty

seconds at the start of a CD, but it can also be any number greater

than 150.  A commmon number is often 183.

The /ID option allows you to manually set the SCSI ID of the CDROM

drive you want to use.  This is useful if my software incorrectly

guesses your drive's ID, or if you have more than one CDROM drive

in your system.

The /MSC option allows you to manually set the MSCDEX drive letter of

the CDROM drive you want to use.  This is useful if my software incorrectly

guesses your drive's ID, or if you have more than one CDROM drive

in your system

The /M option forces the software to use only MSCDEX commands.  This

may or may not work with your drive.  If you don't have a SCSI CDROM

drive with ASPI drivers, this is your only hope.

For the options /S /E /F which expect a following parameter, remember that

there needs to be a space between the letter and the parameter.  ie if you

enter /S01:10:10, it would come up as an error, but if you entered

/S 01:10:10, it would be correct.  For the MSF mode, there need to

be 2 digits for each of minutes, seconds and frames or it will come up as

an error.  When entering the filename with the /F, don't include an

extension, as the software automagically adds the extension for you.

At present there are only two file formats supported, WAVE and Binary.

There is also the option to dump the data in HEX format out to the

STDOUT device.

The binary format has the samples stored in the order Left LSB, Left MSB,

Right LSB, Right MSB.  The samples are 16 bit 44.1 KHz stereo.

I am not expecting to add options to output files in 8-bit or 22.05 KHz.

This is not as simple as just throwing away samples, as this causes

aliasing in the output files.  Correcting this is way beyond the

scope of this program.

I am not planning to add many more formats because there

are plenty of other programs out there that will do the conversions much

better than I can.  Besides, the whole purpose of this program was to

get the raw data out to the hard disk, not duplicate SOX.

On every CD there is a bit which defines if copying a particular track is

permitted or prohibited.  CDDA checks this bit and will not continue

with the dump to disk.  I have included an override option /O which forces

the user to explicitly specify and to knowingly copy a copy prohibited

song.  Have look at the file ROYALTY included in this distribution,

which was honourably pinched from the program CDGRAB.  It is a list

of most country's contacts for paying royalties.

One thing to remember is that this audio fills up the hard disk fast.  It

takes between 9 and 10 megabytes per minute of music.  Because of this I

put in a check to make sure that you will have enough space to put the

requested samples.  As well there is the /U option which will give you

an estimate of how much disk space will be used without actually dumping

the data to disk.

Examples

--------

CDDA /MSF /S 10:14:36 /E 13:55:11 /F outfile /W

will dump using MSF mode from 10:14:36 to 13:55:11 to a WAV file

CDDA /LBA /S 106232 /E 109443 /F outfile /B

will dump using MSF mode from 106232 to 109443 to a Binary file

CDDA /T 2 /F outfile /W /O

will dump the entire track 2 to a WAV file with the override mode on

CDDA /T 2 /U

will display the estimated disk space for all of track 2

CDDA /T 2 /H

will dump the entire track 2 in HEX to STDOUT

CDDA /T 2 /M /F outfile /W /O

will dump the entire track 2 using MSCDEX interface only

CDDA /T 2 /MSC G /ID 4 /F outfile /W /O

will dump the entire track 2 from MSCDEX drive G, SCSI ID 4 CDROM drive

What is this jitter business?

-----------------------------

Data CDROMs are written with sync marks so that the drive can

accurately random access the disc.  On audio CDs these marks

are absent, as they would normally be accessed sequentially.  Because

of this, most every CDROM drive that can read DA frames cannot

accurately return to the exact same location on the disc every time.

It will usually return within a few samples either way.  It took

a lot of time and thought to write the code to correct for the

"jitter" efficiently.  My first try brought the transfer rate

to a crawl.  I have been refining the process to the point that

it barely has any overhead.

What kind of help is there?

---------------------------

There is very limited help from the command line for CDDA.  You can get

the "USAGE" by just typing CDDA.e.g.:

C:>CDDA

Usage: CDDA /(mode) [/U,H,B,W,M] [O] /S <start> /E <end> /F filename

[/ID <cd-rom scsi id>][/MSC <mscdex drive letter>].

modes are LBA, MSF, T

/MSF - times in minute, second, frame format (MM:SS:FF)

/LBA - times in Logical Block Address format (xxxxxx)

/T   - send whole track to file

/S   - Start time

/E   - End time

/U   - estimated disk usage required for data

/H   - hex dump of sectors to stdout

/B   - write to file in Binary format

/W   - write to file in WAV format

/O   - override copy protection bit

/M   - override ASPI interface and use MSCDEX

/MSC - override MSCDEX find first CD-ROM

/ID  - override ASPI find first CD-ROM

Binary mode extension CDA will automagically added to the filename

Wave format extension WAV will automagically added to the filename

e.g. CDDA /MSF /S 10:14:36 /E 13:55:11 /F outfile /W

e.g. CDDA /LBA /S 106232 /E 109443 /F outfile /B

e.g. CDDA /T 2 /F outfile /W /O

Copying and Registration (does he mean this costs $$$?)

--------------------------------------------

I am allowing full freedom to copy this program.  It IS a fully functional

version.  It is NOT Crippleware!  There is NOT a pro version available for

an unreasonable sum of money.  I'm NOT asking for $ to register this program.

What I am asking is, that if you use this program to let me know (via EMAIL,

snail mail, Carrier Pigeon etc.), AND if your karma so directs you to make

a small donation to a local charity.

I would also like to try and keep all the files together and unchanged

in the archive, so if you are passing this around don't change the files.

In this distribution the files contained are:

CDDA.EXE - the program itself

CDDA.DOC - this document file

ROYALTY - the list of people to whom you should pay royalties

CDROMINF.EXE - the MSCDEX exerciser program

CDROMINF.DOC - the text file describing CDROMINF

MSCTEST.EXE - the test program for MSCDEX only access reading DA

TOSHTEST.EXE - the test program for TOSHIBA drives reading DA

NECTEST.EXE - the test program for NEC drives reading DA

SONYTEST.EXE - the test program for SONY drives reading DA

SCSIPING.EXE - the program which uses ASPI to search for SCSI drives

If you are one of those Shareware houses, I don't want to see this program

available for $6.99 plus shipping and handling.  I don't like seeing

the average Joe getting burned for a 10 cent disk, 1 cent label and 2 cents

worth of labour to make the disk, especially when he can get 600+ Meg

of really good stuff from something like the Simtel CD for $25 (thanks to

Robert Bruce for starting the cheap archive business).

How to get hold of me

---------------------

If you want to mail me about problems or to tell me I'm going to rot in hell

for copying music, put your message in a file and copy it to the NUL device.

If you want to make suggestions or want to send compliments you can contact

me at the address below.

If you encounter trouble with the program CDDA, try and run the included

test program most suited to your drive.  There are four test programs:

one for each of the Toshiba, Sony and NEC type drives, and one which

is a MSCDEX only version.  These programs are scaled down versions of

CDDA and use the command line to specify the drive letter and ID.

There is also the program SCSIPING.  It searchs for are reports all devices

connected to the SCSI controller.  It uses the ASPI interface.

Please include any dumps from the programs.  You should be able to

run the programs like this and get the dumps in a text file:

CDDA /T 2 /W /O /F testfile >dumpfile.txt

I have real difficulty in tracking down problems without these

dumps.

EMAIL: jmclaugh@bnr.ca

Snail Mail:

Jim McLaughlin

449 Viewmount Dr.

Nepean, Ont.

Canada

K2E 7P1

Things on my wish list of new functions

---------------------------------------

I want to add support for other CDROM drives that can read DA sectors

over the SCSI, but I don't have access to these drives.  If someone could

supply me with the programming information, I could try and do up a new

version of CDDA, if I ever get some spare time.

Aren't there other programs which do the same thing?

----------------------------------------------------

Yes.  There are several other programs out there which do some or all

of what CDDA does.  In the spirit of the Internet, here are all (I think)

the other programs which read DA frames.

CDDA.ZIP - ftp.cdrom.com - contact: jmclaugh@bnr.ca (my old version)

DA2WAV1A.ZIP - ftp.cdrom.com contact: jmclaugh@bnr.ca (my new version)

CDGRAB32.ZIP - ftp.cdrom.com - contact: cdgrab@aldigital.algroup.com

CDAR010.ZIP - wuarchive.wustl.edu - contact: rhofboer@knoware.nl

NECTOWAV.ZIP - ftp.cdrom.com - contact: zerucha@shell.portal.com

CDINFO.ARJ - archive.utoronto.edu - contact: hpverwei@cs.ruu.nl

CDDAREAD.ZIP - archive.utoronto.edu - contact: ap@jyu.fi

??????.??? - posted to alt.cd-rom - contact yenchee@Jupiter.cdie.nctu.edu.tw

CDDA2WAV.ZIP - ftp.cdrom.com - contact: heiko@colossus.escape.de

READCDA.ZIP - ftp.cdrom.com - contact: hst@mh.nl

Questions I have received since the release of 0.9a

---------------------------------------------------

Why is the program so slow?

This program is so slow because of the following:

1. My poor programming skills.

2. Many of the CDROM drives only read DA frames in 1X mode even though

they may transfer data at 2X or 3X.

3. The amount of computing to do jitter correction.

4. Reordering the bytes from the CD to thw WAV file.

What is ASPI?

ASPI stands for Advanced SCSI Programming Interface.  It was developed

by Adaptec, and has been adopted by many SCSI card manufacturers as

a standard SCSI programming interface.  If you want more information

on ASPI call Adaptec (408) 945-8600.

When I use /T, the start and ends of the track are a couple of

seconds away from where I think they should be.  Why is this?

In version 0.9a, there was a deep rooted bug which caused this.  I

have fixed it (I hope), and this should no longer be a problem.

When I play back my samples, why are there pops and clicks?

These pops and clicks can come from two places:

1. version 0.9a did NOT handle the jitter problem, it is handled now.

2. your sound card can't handle 16-bit 44.1 KHz samples.  Try using SOX

or a similar program to change the sample to 22.05 or 11.025 KHz

samples and try again.

The program stops with the error xxx.  What is wrong?

The 0.9a release had very limited error reporting.  I thought that

if it worked on my machine it work on everybody's.  This version

has much more error reporting.  As I understand it, CDDA will

not necessarily work correctly if SMARTDRV is installed.  This has

been the source of a lot of trouble.  As well some people have been

having trouble using CDDA in a DOS window in Windows.  I think

there may be a problem when being used with a DPMI manager installed.

So, the bottom line is DON'T run CDDA under Windows.

When will you release an OS/2 version?

I have no interest in writing a version for OS/2. Period.  However,

There is a program from Adaptec called VASPI, which will allow

you to run CDDA in a DOS window.  I got the following email with

the how-to for OS/2 from Stefan Eichner (Stefan-Eichner@k2.maus.de):

> 1. Your CONFIG.SYS has to include the follow lines:

> [...]

> BASEDEV=TMV1SCSI.ADD

> BASEDEV=OS2ASPI.DMD

> REM DEVICE=C:\OS2\OS2CDROM.DMD /Q

> E=C:\OS2\MDOS\VASPI.SYS

> [...]

> !If no OS2CDROM.DMD is loaded you can't read Data-CDs!

>

> 2. Add to the autoexec.bat the start-comand for MSCDEX2.EXE

> (e.g. C:\MSCDEX2.EXE /D:MSCD001 /L:H /m:10 /v)

>

> 3. Change the settings of the DOS-Session on the WPS:

> DOS_DEVICE=C:\TSLRCDR.SYS /d:mscd001 /p:3

>

> Now CDDA works great!!!

VASPIBET.ZIP is available from ftp.cdrom.com

When will you release a Windows version?

I have no interest in writing a Windows version.  I don't have the time,

and I don't think Windows is a particularly good OS (if you can call

it an OS).  I have had a couple of offers to help, but I have been

too busy with the rest of my life to work on it.  Don't hold your

breath.

When will you release a UNIX version?

Never.  Although I run LINUX on my machine, I have no interest in

rewriting CDDA.

When will you be writing a version for my CDROM drive?

Getting the programming information from CDROM drive manufacturers is

like pulling teeth.  I've been hunting for this stuff for almost two

years, and have only managed to get the programming manuals for

Toshiba and Sony.  If you have a drive and it does support DA frames,

I need the programming manual.  I also need about 5 or 6 extra hours

in each day.

I have a Toshiba drive.  Why doesn't your program work?

It appears that different ROM revisions cause the drives to work

differently.  For instance, my Toshiba 3401 is revision 0283.  I have

heard from people who have ROMs older and newer that can't use

my program.  I don't know why.

Why doesn't your program work with my ASPI driver?

Several vendors supply ASPI drivers for their SCSI cards.  However,

all drivers are not created equal.  I have heard from people who

have had trouble with just about every SCSI card on the planet.  I

only have Adaptec's EZSCSI to do my testing.  I'll try and do my

best on case by case basis, but I really don't have any spare time.

CDROMINF works on my machine, so why doesn't CDDA?

CDROMINF only uses MSCDEX calls, and CDDA also uses ASPI calls.  It

is these ASPI calls that do the meat of the program.  If ASPI isn't

on your machine, or doesn't work correctly, then CDDA will never work.